Movement Classification

Accelerometer Data Logging & Machine Learning with micro:bit

# Overview

In this project, you will use the BBC micro:bit's built-in accelerometer to record motion data, then train a machine learning model to classify whether the device is being held steady or actively shaken. This is a hands-on introduction to the full data pipeline in biomedical engineering: sensor data acquisition, data transfer, preprocessing, and model training.

The project has three main tasks:

- Task 1: Write a MakeCode or Python script to log accelerometer data from the micro:bit to your PC (via USB cable or radio connection).

- Task 2: Record a structured movement sequence (~20 seconds) using your script.

- Task 3: Use a provided Jupyter Notebook to train a classifier (e.g., logistic regression) that distinguishes steady from shaking movement, then report your accuracy and suggest improvements.

💡 Tip: Before starting, read through the recommended tutorials: Movement data logger, Python data logger, Sensitive step counter, and User guide – Data logging. These are linked from the project README and will give you important context for Tasks 1 and 2.

# Prerequisites

## Required Hardware

- BBC micro:bit (v1 or v2)

- USB cable (micro-USB for v1, USB-C for v2) for wired data transfer

- Optional: a second micro:bit if you plan to use radio (wireless) data transfer

## Required Software

- A micro:bit programming environment, either MakeCode (browser-based, no install needed) or the Mu Editor / VS Code for MicroPython

- Python 3.x with a Jupyter Notebook environment (e.g., VS Code with the Jupyter extension, or JupyterLab)

- Python libraries: numpy, pandas, matplotlib, scikit-learn (see Installation below)

## Recommended Background Reading

Work through the following tutorials before starting. They are linked from the project README:

- Movement data logger — how to record accelerometer data on the micro:bit

- Python data logger — how to receive and save data from the micro:bit on your PC

- Sensitive step counter — example of processing accelerometer data for activity detection

- User guide – Data logging — overview of the micro:bit's built-in data logging feature (v2 only)

📝 Note: The built-in data logging feature (User guide – Data logging) is only available on micro:bit v2. If you have v1, you will need to write a custom serial logging script instead — see Task 1 options below.

# Installation & Setup

## Step 1: Set Up a Python Environment

Open Anaconda Prompt (Windows) or Terminal (macOS/Linux) and create a dedicated environment:

conda create --name movement_cls python=3.10

conda activate movement_cls

## Step 2: Install Required Python Libraries

pip install numpy pandas matplotlib scikit-learn

If you plan to read data from the micro:bit over a serial (USB) connection in Python, also install:

pip install pyserial

To run the Jupyter Notebook, install Jupyter as well:

pip install jupyter

💡 Tip: If you use VS Code with the Jupyter extension, you may not need to install jupyter separately. VS Code can manage this for you. Make sure to select the movement_cls conda environment as your kernel when opening the notebook.

## Step 3: Set Up Your micro:bit Programming Environment

Choose one of the two approaches below based on your preference:

Option A: MakeCode (Beginner-friendly, Block or JavaScript)

- Open your browser and go to: https://makecode.microbit.org/

- Click New Project to start a new script.

- You can write your data logging script in Blocks, JavaScript, or Python (MakeCode Python).

- To flash your script to the micro:bit, connect it via USB and click Download, then drag the .hex file onto the micro:bit drive that appears.

Option B: MicroPython with Mu Editor or VS Code

- Download and install the Mu Editor from: https://codewith.mu/ -  it has built-in micro:bit support and a serial plotter/monitor.

- Alternatively, use VS Code with the MicroPython extension.

- Connect your micro:bit via USB. In Mu, select BBC micro:bit mode, write your script, and click Flash.

📝 Note: MakeCode and MicroPython are two different environments with different syntax. The tutorials linked in the README may show one or the other — pick the approach that matches the tutorial you are following.

# Task 1: Write an Accelerometer Data Logger

Your goal is to write a script that continuously reads X, Y, and Z acceleration values from the micro:bit's accelerometer and saves them to your PC. You have two options for data transfer: wired (USB serial) or wireless (radio).

## Option A: Wired Transfer via USB Serial

This is the simpler and more reliable option. The micro:bit sends accelerometer readings over USB serial, and a Python script on your PC listens and saves them to a file.

Part 1: micro:bit Script

Write a MakeCode or MicroPython script on the micro:bit that:

- Reads the accelerometer's X, Y, and Z values in a loop.

- Prints them as comma-separated values over serial (e.g., x,y,z).

- Includes a timestamp or simply relies on the PC to timestamp on receipt.

Example MicroPython script for the micro:bit:

from microbit import *

import utime

uart.init(115200)

while True:

x = accelerometer.get_x()

y = accelerometer.get_y()

z = accelerometer.get_z()

print(f'{x},{y},{z}')

utime.sleep_ms(50)  # ~20 Hz sampling rate

Part 2: PC-side Python Logger

Write a Python script on your PC that reads from the serial port and saves the data to a CSV file:

import serial, time

port = 'COM3'   # Windows: COM3, COM4, etc.

# macOS/Linux: '/dev/tty.usbmodem...' or '/dev/ttyACM0'

baud = 115200

with serial.Serial(port, baud, timeout=1) as ser:

with open('movement_data.csv', 'w') as f:

f.write('timestamp,x,y,z\n')

start = time.time()

while True:

line = ser.readline().decode('utf-8').strip()

if line:

t = round(time.time() - start, 3)

f.write(f'{t},{line}\n')

print(f'{t}: {line}')

💡 Tip: To find your serial port on Windows: open Device Manager and look under 'Ports (COM & LPT)' for a USB Serial Device. On macOS/Linux: run ls /dev/tty.* or ls /dev/ttyACM* in terminal.

## Option B: Wireless Transfer via Radio

If you have two micro:bits, one can act as a sensor (worn or held) and transmit data wirelessly to a second micro:bit connected to your PC, which forwards the data over serial.

Sender micro:bit Script

from microbit import *

import radio, utime

radio.on()

radio.config(channel=7)

while True:

x = accelerometer.get_x()

y = accelerometer.get_y()

z = accelerometer.get_z()

radio.send(f'{x},{y},{z}')

utime.sleep_ms(50)

Receiver micro:bit Script (connected to PC via USB)

from microbit import *

import radio

radio.on()

radio.config(channel=7)

while True:

msg = radio.receive()

if msg:

print(msg)

Then use the same PC-side Python logger from Option A to save the received data to a CSV file.

📝 Note: Both micro:bits must use the same radio channel number (e.g., channel=7) for communication to work.

## Option C: micro:bit v2 Built-in Data Logging

If you have a micro:bit v2, you can use the built-in data logging feature which saves data directly to the micro:bit's flash storage as a CSV file, which you then copy to your PC.

Refer to the 'User guide – Data logging' tutorial linked in the README for full instructions on this method.

# Task 2: Record the Movement Sequence

Once your data logger is working, record the following structured movement sequence. Aim for approximately 20 seconds of total data.

| Step | Action | Duration | Expected Data Pattern |
| --- | --- | --- | --- |
| 1 | Hold the micro:bit completely steady | 5 seconds | Low, constant X/Y/Z values (near 0,0,1024) |
| 2 | Shake the micro:bit vigorously | 5 seconds | High-variance, rapidly fluctuating values |
| 3 | Hold the micro:bit completely steady | 5 seconds | Low, constant X/Y/Z values again |
| 4 | Shake the micro:bit vigorously | 5 seconds | High-variance, rapidly fluctuating values |

## Step-by-Step Recording Instructions

- Start your PC-side logger script (or start data logging on the micro:bit v2).

- Hold the micro:bit as still as possible for 5 seconds. Rest it on a flat surface if needed.

- Shake the micro:bit vigorously in random directions for 5 seconds.

- Hold it still again for 5 seconds.

- Shake it again for 5 seconds.

- Stop the logger script. You should have approximately 20 seconds of data (around 400 rows at 20 Hz).

⚠️  Important: You must manually add a label column to your CSV file after recording, indicating which rows correspond to 'steady' and which correspond to 'shaking'. You can do this based on the timestamps. For example, rows 0–5 seconds = steady, 5–10 seconds = shaking, etc.

💡 Tip: Plot your data immediately after recording (using matplotlib or the serial plotter in Mu Editor) to visually verify the steady and shaking segments look as expected before moving to Task 3.

## Labeling Your Data

Open your CSV file and add a label column. You can do this in Python:

import pandas as pd

df = pd.read_csv('movement_data.csv')

# Label based on timestamp (adjust boundaries to match your recording)

df['label'] = 'steady'

df.loc[(df['timestamp'] >= 5) & (df['timestamp'] < 10), 'label'] = 'shaking'

df.loc[(df['timestamp'] >= 15) & (df['timestamp'] < 20), 'label'] = 'shaking'

df.to_csv('movement_data_labeled.csv', index=False)

# Task 3: Train a Movement Classifier

Open the provided Jupyter Notebook (linked as 'this notebook' in the README) and follow the steps to train a classifier that distinguishes between steady and shaking movement.

## General Notebook Workflow

- Load your labeled CSV data into a pandas DataFrame.

- Extract features from the raw X, Y, Z accelerometer signals (e.g., rolling mean, standard deviation, signal magnitude).

- Split the data into training and test sets.

- Train a classifier, the notebook uses logistic regression as a default example.

- Evaluate the model and report classification accuracy.

## Feature Engineering

Raw accelerometer values alone may not be the most informative features. Consider computing the following over a sliding window:

- Signal Magnitude Vector (SMV): sqrt(x² + y² + z²): a single value capturing total movement intensity

- Rolling standard deviation of X, Y, or Z, high during shaking, low when steady

- Rolling mean: captures average acceleration level

- Min/max range within a window, large during shaking

💡 Tip: A window size of 20 - 50 samples (1 - 2.5 seconds at 20 Hz) often works well for activity classification tasks like this one.

## Classifier Options

The notebook uses logistic regression as a baseline, but you are free to try other classifiers. Here are some options available in scikit-learn:

| Classifier | scikit-learn Class | Notes |
| --- | --- | --- |
| Logistic Regression | LogisticRegression() | Baseline: fast, interpretable, works well on linearly separable data |
| Decision Tree | DecisionTreeClassifier() | Easy to visualize and interpret; can overfit |
| Random Forest | RandomForestClassifier() | Often higher accuracy; ensemble of decision trees |
| Support Vector Machine | SVC() | Strong classifier for small/medium datasets |
| k-Nearest Neighbors | KNeighborsClassifier() | Simple, no training phase; slower at prediction |
| Rule-based (custom) | if/else logic on SMV | No ML needed, e.g., if SMV > threshold: shaking |

💡 Tip: A rule-based system (e.g., if SMV > 1200 then 'shaking') can be surprisingly effective for simple binary motion classification and requires no training data. This is a valid approach for this project.

## Evaluating Your Model

After training, evaluate your model on the held-out test set. Report the following in your submission:

- Classification accuracy (percentage of correctly classified windows)

- (Optional) Confusion matrix: shows how many steady windows were misclassified as shaking and vice versa

- (Optional) Precision, recall, and F1-score

Example evaluation code:

from sklearn.metrics import accuracy_score, confusion_matrix

y_pred = model.predict(X_test)

print('Accuracy:', accuracy_score(y_test, y_pred))

print('Confusion Matrix:')

print(confusion_matrix(y_test, y_pred))

## Suggestions for Improving Accuracy

In your written submission, you are asked to suggest improvements. Consider discussing the following:

- Collect more data: more recordings of both classes improves generalization

- Better features: try different window sizes or additional features like frequency-domain features (FFT)

- Balance the classes: ensure roughly equal amounts of steady and shaking data

- Try a more powerful model: e.g., Random Forest or SVM instead of logistic regression

- Tune hyperparameters: use cross-validation or grid search to find optimal model settings

- Reduce noise: apply a smoothing filter to the raw accelerometer signal before feature extraction

# Submission Requirements

Submit the following items through your course portal:

## 1. Written Document (1 Word file)

- Classification accuracy achieved by your model (or rule-based system).

- A brief description of which classifier/approach you used and why.

- At least 3 concrete suggestions for how the classification accuracy could be improved, with brief justifications.

## 2. Data File

- Your labeled CSV file (movement_data_labeled.csv) containing the ~20 seconds of recorded accelerometer data with the label column.

## 3. Completed Jupyter Notebook

- The provided notebook with all cells executed and outputs visible.

- Your chosen model trained and evaluated on your recorded data.

- Accuracy and (optionally) confusion matrix displayed in the output.

# Troubleshooting

| Problem | Solution |
| --- | --- |
| micro:bit not recognized as a drive | Try a different USB cable, some cables are charge-only and do not support data transfer. Try a different USB port on your PC. |
| Serial port not found / permission denied | On macOS/Linux, run: ls /dev/tty.* to find the port. On Linux, you may need to add yourself to the 'dialout' group: sudo usermod -aG dialout $USER (then log out and back in). |
| No data received by PC logger | Check that the baud rate in your Python script matches the baud rate in the micro:bit script (both should be 115200). Also ensure the micro:bit script has been flashed successfully. |
| Radio connection unreliable / data loss | Reduce distance between micro:bits. Ensure both use the same channel. Switch to wired USB transfer if reliability is important. |
| ModuleNotFoundError: pyserial | Run: pip install pyserial in your active conda environment. |
| Low classification accuracy (< 70%) | Check that your labels are correctly aligned with timestamps. Plot the data to visually verify steady vs. shaking segments. Try adding the SMV feature or increasing window size. |
| Notebook kernel not found | Select the movement_cls conda environment as the kernel. In VS Code: click 'Select Kernel' at the top right of the notebook. |
| Data file has wrong number of rows | Verify your sampling rate. At 50 ms per sample (20 Hz), 20 seconds should give ~400 rows. Adjust the sleep_ms value in your micro:bit script if needed. |

# Quick Reference — Command Cheat Sheet

Environment setup (run once):

conda create --name movement_cls python=3.10

conda activate movement_cls

pip install numpy pandas matplotlib scikit-learn pyserial jupyter

Activate environment (every session):

conda activate movement_cls

Find serial port on macOS/Linux:

ls /dev/tty.*          # macOS

ls /dev/ttyACM*        # Linux

Convert notebook to script (if needed):

jupyter nbconvert --to script notebook_name.ipynb

Launch Jupyter Notebook:

jupyter notebook

# Additional Resources

- CI-BioEng-Class Repository: https://github.com/cyneuro/CI-BioEng-Class

- micro:bit MakeCode Editor: https://makecode.microbit.org/

- Mu Editor (MicroPython for micro:bit): https://codewith.mu/

- MicroPython micro:bit Documentation: https://microbit-micropython.readthedocs.io/

- micro:bit Data Logging Guide (v2): https://microbit.org/get-started/user-guide/data-logging/

- scikit-learn User Guide: https://scikit-learn.org/stable/user_guide.html

- pyserial Documentation: https://pyserial.readthedocs.io/

- Anaconda Download: https://www.anaconda.com/download/success